Water Resources Review Sheet

Final Exam

Dr. Zada Tawalbeh

Question 1: Multiple Choice

1) The hydrologic cycle mainly involves:
A) Condensation, filtration, and percolation
B) Precipitation, evaporation, runoff, and infiltration
C) Transpiration only
D) Groundwater recharge only

Answer: B

2) Drainage basin, catchment, and watershed are:
A) Different hydrologic systems
B) Synonymous terms
C) Only used in urban hydrology
D) Related only to groundwater

Answer: B

Precipitation Processes

3) Convectional precipitation occurs due to:
A) Mountain barriers
B) Heating of the Earth’s surface
C) Collision of warm and cold fronts
D) High atmospheric pressure

Answer: B

4) Orographic precipitation occurs on the:
A) Leeward side of mountains
B) Windward side of mountains
C) Desert side of basins
D) Downstream side of rivers

Answer: B

5) Cyclonic (frontal) precipitation results from:
A) Soil heating
B) Air turbulence
C) Interaction of warm and cold air masses
D) Groundwater discharge

Answer: C

6) A rainfall hyetograph represents:
A) Rainfall depth vs. area
B) Rainfall intensity vs. time
C) Runoff vs. time
D) Infiltration vs. soil type

Answer: B

7) The Arithmetic Mean Method assumes:
A) Unequal gauge weights
B) Uniform gauge distribution
C) Orographic influence
D) Variable rainfall intensity

Answer: B

8) A disadvantage of the Thiessen Polygon method is:
A) Difficulty in calculations
B) Does not account for orographic effects
C) Requires no map
D) Assumes equal rainfall everywhere

Answer: B

Infiltration & Hydrologic Modeling

9) Infiltration is defined as:
A) Water flowing over land
B) Downward movement of water into soil
C) Evaporation from soil
D) Groundwater discharge

Answer: B

10) Infiltration generally decreases with time because:
A) Rainfall stops
B) Soil becomes saturated
C) Temperature decreases
D) Wind speed increases

Answer: B

11) A hydrologic budget is based on:
A) Energy conservation
B) Momentum conservation
C) Mass balance
D) Heat transfer

Answer: C

12) Evaporation is:
A) Condensation of water vapor
B) Conversion of liquid water to vapor
C) Infiltration into soil
D) Runoff generation

Answer: B

13) Evaporation rate increases with:
A) High relative humidity
B) Low temperature
C) High wind speed
D) High salinity

Answer: C

14) Evaporation is inversely proportional to:
A) Temperature
B) Wind speed
C) Salinity
D) Surface area

Answer: C

15) The pan evaporation correction factor (K) typically ranges between:
A) 0.1–0.3
B) 0.4–0.5
C) 0.6–0.8
D) 0.9–1.0

Answer: C

Evapotranspiration (ET)

16) Evapotranspiration consists of:
A) Evaporation only
B) Transpiration only
C) Evaporation and transpiration
D) Infiltration and runoff

Answer: C

17) Which factor does NOT affect evapotranspiration?
A) Crop type
B) Soil moisture
C) Sunshine hours
D) Water hardness

Answer: D

18) A lysimeter is used to estimate:
A) Rainfall
B) Infiltration
C) Evapotranspiration
D) Runoff

Answer: C

19) Which method uses climatic data to estimate ET?
A) Hargreaves method
B) Thiessen method
C) Isohyetal method
D) Horton method

Answer: A

20) Relative Humidity (RH) is defined as:
A) Amount of rainfall in the air
B) Ratio of actual vapor pressure to saturation vapor pressure
C) Total evaporation rate
D) Percentage of cloud cover

Answer: B

21) When RH equals 100%, the air is:
A) Completely dry
B) Unsaturated
C) Fully saturated
D) Unable to condense

Answer: C

22) At 100% RH, water vapor begins to:
A) Evaporate rapidly
B) Condense into liquid
C) Increase wind speed
D) Reduce air pressure

Answer: B

23) Evaporation rate increases with:
A) Low temperature
B) High relative humidity
C) Strong wind speed
D) High salinity

Answer: C

24) Evaporation is inversely proportional to:
A) Temperature
B) Wind speed
C) Area of water surface
D) Salinity of water

Answer: D

25) Larger exposed water surface area results in:
A) Lower evaporation
B) No change in evaporation
C) Higher evaporation
D) Condensation

Answer: C

26) Evaporation rate is higher when atmospheric pressure is:
A) High
B) Constant
C) Low
D) Zero

Answer: C

27) The pan evaporation method requires a:
A) Runoff coefficient
B) Pan coefficient
C) Crop coefficient
D) Soil factor

Answer: B

28) Evapotranspiration includes:
A) Evaporation only
B) Transpiration only
C) Evaporation and transpiration
D) Infiltration and runoff

Answer: C

29) Which of the following does NOT affect evapotranspiration?
A) Crop type
B) Soil moisture
C) Wind speed
D) Water salinity

Answer: D

30) Which method directly measures evapotranspiration?
A) Thiessen method
B) Isohyetal method
C) Eddy covariance method
D) Arithmetic mean method

Answer: C

31) Steady flow is defined as flow where:
A) Velocity changes with time
B) Density changes rapidly
C) Flow properties do not change with time
D) Flow occurs intermittently

Answer: C

32) An example of unsteady flow is:
A) Water in a constant diameter pipe
B) Tidal currents
C) Laminar pipe flow
D) Groundwater seepage

Answer: B

33) Incompressible flow is characterized by:
A) Large density changes
B) Constant density
C) High velocity gas flow
D) Shock waves

Answer: B

34) Kinematic viscosity is defined as:
A) Ratio of density to pressure
B) Ratio of dynamic viscosity to density
C) Product of velocity and area
D) Resistance to compression

Answer: B

35) Capillarity occurs due to the interaction of:
A) Gravity and pressure
B) Wind and temperature
C) Cohesion and adhesion forces
D) Inertia and viscosity

Answer: C

36) Capillary rise occurs when:
A) Cohesion force > adhesion force
B) Adhesion force > cohesion force
C) Both forces are equal
D) Gravity is dominant

Answer: B

37) Mercury in a glass tube shows:
A) Capillary rise
B) Capillary fall
C) No capillary effect
D) Evaporation

Answer: B

Question2: True/False questions

- Runoff is a fundamental component of the hydrologic cycle.
True

- Runoff occurs only when rainfall intensity is less than infiltration capacity.
False

- Runoff can occur when soil is completely saturated.
True

- Precipitation (P) represents the total accumulated rainfall over a given time.
True

- Initial abstraction (Ia) includes interception, surface storage, evaporation, and initial infiltration.
True

- Continuing abstraction (Fa) occurs before runoff begins.
False

- Precipitation excess (Pe) is the portion of rainfall that forms direct runoff.
True

- The rainfall–runoff relationship can be expressed as:
P = Pe + Ia + Fa
True

- Runoff estimation is not required for flood analysis.
False

- Rainfall–runoff modeling is used to estimate streamflow from rainfall data.
True

- The unit hydrograph method is one technique used to estimate flood peaks.
True

- The time-area method is used for estimating runoff discharge.
True

- The SCS Curve Number method is an empirical runoff estimation method.
True

- The rational method is suitable for very large catchments.
False

- The rational method is generally applicable to catchments smaller than 50 km².
True

- The unit hydrograph method is commonly used for catchments up to about 5000 km².
True

- Specific gravity is the ratio of a fluid’s density to the density of air.
False

- Specific gravity of water is equal to 1.
True

- Vacuum pressure is pressure higher than atmospheric pressure.
False

- Capillary rise occurs when cohesion forces are stronger than adhesion forces.
False

- Laminar flow is characterized by smooth and orderly fluid motion.
True

- Turbulent flow has random velocity fluctuations.
True

Notes:

You must pay close attention and practice the calculation questions for different Rainfall-Runoff Methods.

You must learn the main concepts of runoff estimation methods.