| T103/TM103 Computer Organization and Architecture |  |
| --- | --- |
| Midterm Examination (MTA) Solution Fall Semester 2023/2024 Date: ?? /xx./2023 |  |
| Time Allowed: 1.5 Hours | Number of Exam Pages: 4 (including this cover sheet) |

Instructions:

- Total Marks: 60

- This exam consists of two parts.

- ALL questions must be answered in the External Answer booklet.

- Be sure you write your name and ID on the External Answer booklet.

- Programmable Calculators are not allowed.

### Part 1:  Multiple Choices questions (5 x 2 = 10 Marks)

### Answer all the following questions by choosing the most correct statement

- For the AND gate, the output will be 0 if:

- All inputs are 1, exclusively.

- All inputs are 0, exclusively.

- Only when inputs have different values.

- None of the choices.

- Which 2-input logic gate produces a 1 output when any one of its inputs is 1, but not both?

- AND.

- XOR.

- NOR.

- NAND.

- In the SR-Flip Flop, if R=0, S=1 then:

  - Q(t+1) is unknown.

  - Q(t+1)= Q’(t)

  - Q(t+1)= 1

  - None of the choices.

- What decimal value does the 8-bit binary number 10111010 represent, if the computer is using a C1 representation?

- -58.

- 58.

- -69.

- -186.

- A Multiplexer with 60 inputs needs at least:

- 8 selectors

- 4 selectors

- 16 selectors

- 6 selectors.

PART 2: ALL QUESTIONS ARE REQUIRED [50 Marks]

Remark for questions in Part 2:

All the answers below for the questions are considered ideal answers to be considered in correction. However, the tutor should expect student’s answers to be provided in a different style and format. Kindly consider these answers and allow for partial marks to be awarded accordingly.

Q-1: Using truth table, check whether   F=  x’+yz’, and G= x(y’+z) are complements to each other or not.                                                                                                               [6 marks]

Answer: (6 marks: 2.5 marks for each T.T; 1 mark for the conclusion)

| Inputs |  |  | Intermediate |  |  | F | Intermediate |  | G |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| x | y | z | x’ | z’ | yz' | x' +yz' | y’ | y'+z | x(y'+z) |
| 0 | 0 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 0 |
| 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 0 |
| 0 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 0 |
| 0 | 1 | 1 | 1 | 0 | 0 | 1 | 0 | 1 | 0 |
| 1 | 0 | 0 | 0 | 1 | 0 | 0 | 1 | 1 | 1 |
| 1 | 0 | 1 | 0 | 0 | 0 | 0 | 1 | 1 | 1 |
| 1 | 1 | 0 | 0 | 1 | 1 | 1 | 0 | 0 | 0 |
| 1 | 1 | 1 | 0 | 0 | 0 | 0 | 0 | 1 | 1 |

F and G columns show that the two functions have opposite outputs, Hence, G is the complement of F.

Q-2: Given the circuit below that implements the Boolean expression F.            [6 marks]

    - Write the equation of F.

    - What will be the value at the output if the input values are: A=0, C=1? Justify your answer.

Answer: (6 marks: 3 +3)

- Fx, y=A.C   , or (A’.C)  (3 marks)

- The output will be 1 (2 marks), because A’=1 and C=1 then A’C=1.   (1 mark)

Q-3: Represent the decimal number (-79) in:                                             [11 marks]

- Signed Magnitude 8-bit representation.

- Two’s Complement 8-bit representation.

- Excess-16 Floating point representation.

| 8-bit signed magnitude | 11001111 |  |  |  |  |  |  |  |  |  |  |  |  |  | [3 marks] |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| two’s complement | 10110001 |  |  |  |  |  |  |  |  |  |  |  |  |  | [3 marks] |  |
| Excess-16 FP | 1 | 1 | 0 | 1 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | 1 | 0 |  | [5 marks] |  |

Q-4: Given the following F in its Sum of Product form:                                     [12 marks]

F(x,y,z)SoP= x’y’z’ + x’yz  + x’yz’+ xy’z’  + xyz + xyz’

- Draw the K-Map of the function “F”.

- Show the grouping that you selected for simplifying the function.

- Write the simplified F.

Answer:

- (4 marks)

- (4 marks for the proper grouping)

- F= y + z’ (4 marks: Deduct 1 mark if the student added a redundant term)

Q-5: Perform the following base conversions:                                             [15 marks]

- (116.25)10        =  (1110100.01)2         [3 marks]

- (11001101.110)2 = (315.6)8         [3 marks]

- (3D.C)16       = (61.75)10    [3 marks]

- (3D.C)16   =  (111101.11)2 = (75.6)8  [6 marks: 3+3]

___________________________________________________________________________

End of questions