| T103/TM103 Computer Organization and Architecture |  |
| --- | --- |
| Midterm Examination (MTA) Make-up Solution Fall Semester 2023/2024 Date: ?? /xx./2023 |  |
| Time Allowed: 1.5 Hours | Number of Exam Pages: 4 (including this cover sheet) |

Instructions:

- Total Marks: 60

- This exam consists of two parts.

- ALL questions must be answered in the External Answer booklet.

- Be sure you write your name and ID on the External Answer booklet.

- Programmable Calculators are not allowed.

### Part 1:  Multiple Choices questions (5 x 2 = 10 Marks)

### Answer all the following questions by choosing the most correct statement.

- For the NAND gate, the output will be 0 if:

- All inputs are 1, exclusively.

- All inputs are 0, exclusively.

- Any of its inputs is 0.

- Any of its inputs is 1.

- Which logic gate produces a 1 output only when its two inputs are different?

- AND.

- XOR.

- NOR.

- NAND.

- In the JK-Flip Flop, if K=0, J=1 then:

  - Q(t+1) is unknown.

  - Q(t+1)= Q’(t)

  - Q(t+1)= 1

  - None of the choices.

- What decimal value does the 8-bit binary number 10111010 represent , if the computer is using a C2 representation?

- -58.

- 58.

- -70.

- -186.

- What is the equivalent value in microseconds for the value 15*10-2 ms?

- 15*10-5

- 15*103

- 0.15*10-3

- 15*101.

PART 2: ALL QUESTIONS ARE REQUIRED [50 Marks]

Remark for questions in Part 2:

All the answers below for the questions are considered ideal answers to be considered in correction. However, the tutor should expect student’s answers to be provided in a different style and format. Kindly consider these answers and allow for partial marks to be awarded accordingly.

Q-1: Draw the truth table of the half adder and write the equation of the Sum and the Carry outputs.                                                                                                                   [6 marks]

Answer: (6 marks)

(4 marks:2 for the sum, 2 for the carry)

Sum = 𝑥⊕𝑦, Carry = xy (2 marks: 1 +1)

Q-2: Represent the decimal number (-26.75) in floating point notation excess16.  [7 marks]

Answer: (7 marks: 1 mark for the sign, 3 marks for each of the exponent and the significand)

| FP-excess 16 | 1 | 1 | 0 | 1 | 0 | 1 | 1 | 1 | 0 | 1 | 0 | 1 | 1 | 0 | [7 marks] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Q-3: Find the equivalent decimal number of the signed binary number 11100011 if the system uses signed magnitude, one’s complement and two’s complement                          [9 marks]

Answer: [9 marks: 4+3+2]

| signed magnitude: | The equivalent decimal number is -99 |
| --- | --- |
| one’s complement | The equivalent decimal number is -28 |
| two’s complement | The equivalent decimal number is -29 |

Q-4: Given the following truth table:                                                                           [13 marks]

- Deduce from the above table, the Sum of Product form of F.

- Draw the K-map of the function F, showing the groupings you have selected.

- Deduce the simplified F from the K-map.

- Draw the logic gate representation (diagram) of the simplified F.

Answer:

- (4 marks: 1 mark for each minterm)

F(x,y,z)= x’yz + xy’z’ + xyz’ + xyz

- (4 marks: 2 marks for the proper positioning of 1’s; 2 marks for proper grouping)

- F= yz + xz’ (3 marks: Deduct 1 mark if the student added a redundant term)

- Logic diagram (2 marks, based on the result in part c)

Q-5: Perform the following base conversions:                                                          [15 marks]

- (138.5)10        =  (212.4)8         [3 marks]

- (1011010.01)2 = (132.2)8         [3 marks]

- (10E.4)16       = (270.25)10    [3 marks]

- (3D.C)16   =  (111101.11)2 = (75.6)8  [6 marks: 3+3]

___________________________________________________________________________

End of questions