| T103/TM103 Computer Organization and Architecture |  |
| --- | --- |
| Midterm Examination (MTA) Solution Spring Semester 2023/2024 Date: ?? /???/2024 |  |
| Time Allowed: 1.5 Hours | Number of Exam Pages: 3 (including this cover sheet) |

Instructions:

- Total Marks: 60

- This exam consists of two  parts.

- ALL questions must be answered in the External Answer booklet.

- Be sure you write your name and ID on the External Answer booklet.

- Programmable Calculators are not allowed.

### Part 1:  Multiple Choices questions (5 x 2 = 10 Marks)

### Answer all the following questions by choosing the most correct statement.

- How many milliseconds (ms) are in 0.5 seconds?

  - 50000

  - 0.005

  - 500

  - None of the choices

- The logical operator "NOR" returns a 1 value when:

  - Exclusively all inputs are 1.

  - Any input is 1.

  - Any input is 0.

  - Exclusively all inputs are 0

- The biggest decimal number that can be represented in a 6-bit unsigned whole numbering system is:

  - 16

  - 31

  - 64

  - 63

- A computer speed is mainly affected by:

- The ROM.

- The number of I/O devices.

- The CPU’s clock speed.

- All the choices.

- What is the complement of the expression F=x+y+z ?

- x. y.z

- x +y+z

- x+y+z

- None of the choices.

________________________________________________________________

PART 2: ALL QUESTIONS ARE REQUIRED [50 Marks]

Remark for questions in Part 2:

Kindly consider any equivalent answers and allow for partial marks to be awarded accordingly.

Q-1: Answer the following questions:                                         [6 marks]

- What is a cache memory?

- Name two types of ports in a computer (whether old or new)

- Name two levels of the computer hierarchy.

Answer: [6 marks: 2 marks for each part]

- Cache is a type of temporary memory that can be accessed faster than RAM

- Any two types of ports. For example, USB, serial, parallel, PS/2…

- Any two levels: User’s level, High-Level Language Level, Assembly Language Level’ System Software Level, Machine Level, Control Level, Digital Logic Level

Q-2: Represent in floating-point excess16 the number -57.7510.             [7 marks]

Answer: (7 marks: 1 mark for the sign, 3 marks for each of the exponent and the significand)

| FP-excess 16 | 1 | 1 | 0 | 1 | 1 | 0 | 1 | 1 | 1 | 0 | 0 | 1 | 1 | 1 | [7 marks] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Q-3: Represent the decimal number (-95) in binary using 8-bit sign magnitude, one’s complement, and two’s complement:                                                  [9 marks]

Answer: [9 marks: 3 marks for each representation]

| 8-bit sign magnitude | 11011111 |
| --- | --- |
| one’s complement | 10100000 |
| two’s complement | 10100001 |

Q-4: Perform the following base conversions:                                             [14 marks]

- (151)10        =  (10010111)2         [2 marks]

- (01100110.01)2 = (146.2 )8  [4 marks: 2 marks for the integer ; 2 marks for the fraction ]

- (2B .4)16    = (43.25)10   [4 marks: 2 marks for the integer part ; 2 marks for the fraction part]

- (674)8  = (1BC) 16 [4 marks: award partial marks if the conversion to binary is correct]

Q-5: Given the following truth table:                                                            [14 marks]

- Deduce from the above table, the Sum of Product form of F.

- Draw the K-map of the function F, showing the groupings you have selected.

- Deduce the simplified F from the K-map.

- Draw the logic gate representation (diagram) of the simplified F.

Answer: (14 marks)

- (3 marks: 0.5 marks for each minterm)

F(a,b,c)= a’b’c’ + a’bc’ + ab’c’ + ab’c + abc’ + abc

- (6 marks: 3 marks for the proper positioning of 1’s; 3 marks for proper grouping)

- F= a + c’ (3 marks: Deduct 1 mark if the student added a redundant term)

- Logic diagram (2 marks)

End of questions