Faculty of Computer Studies

Computer Organization and Architecture

T103/TM103

TMA

Fall 2022-2023

| Cut-off Date: TBA |  | Total Marks: 20 |  |  |
| --- | --- | --- | --- | --- |
| Content |  |  | Mark |  |
| Digital circuits | ∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙ |  | 10 |  |
| Digital circuits | ∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙∙ |  | 10 |  |

Plagiarism Warning:

As per AOU rules and regulations, all students are required to submit their own TMA work and avoid plagiarism. The AOU has implemented techniques for plagiarism detection. You must provide all references in case you use and quote another person's work in your TMA. You will be penalized for any act of plagiarism as per the AOU's rules and regulations.

Declaration of No Plagiarism by Student (to be signed and submitted by student with TMA work):

I hereby declare that this submitted TMA work is a result of my own efforts and I have not plagiarized any other person's work. I have provided all references of information that I have used and quoted in my TMA work.

Name of Student:

Signature:

Date:

Problem 1: Working with Digital Circuits                           (10 Marks)

Suppose that you want to design a new flip-flop and name it as AM flip-flop. This AM flip-flop behaves as follows: If A = 1, the flip-flop complements the current state. If A = 0, the next state of the flip-flop is equal to the value of M.

- Derive the characteristic table for the AM flip-flop.

- From the characteristic tables of both flip-flops, the JK and the AM (that you have derived in part a), find the equivalent values of J & K for each of the AM states.

For example, if for A= 0, M=0, Q(t+1) was, say, x; then you should find the equivalent combination of J & K that produces the same output.

- Based on the result that you have obtained in part b, show how a JK flip-flop can be converted to an AM flip-flop by adding gate(s) and inverter(s).

Problem 2: Working with Digital Circuits                           (10 Marks)

Consider the function: F(x,y,z) = xy+z’ .

- Draw the logical diagram of F.

- Using only NAND gates, represent F.

- Draw the minimized logical representation of the function F found in part b.

End of TMA